For immediate release

Contacts: George May, (603) 883-3409 / georgemay@adelphia.net

Ken Butenhof, (603) 644-3431 / kbutenhof@comcast.net

LOWER HALF OF SOUHEGAN HAS HIGH E. COLI COUNTS

Although southern NH was spared the heavy rains and flooding down south and in upstate New York, high flows in the Souhegan and Merrimack Rivers was the dominant feature noted by volunteers collecting water quality samples on Tuesday morning, June 27th.  Although flows were one-third the level of two weeks ago, the rivers were still very high and the current very fast. The Souhegan was running at 508 cubic feet per second, five times the normal historic flow of 96 for this date; flow on the Merrimack was 11,500 CFS, almost five times its 2540 expected flow. Drizzly weather for the three days prior to the water testing brought the flow on both rivers back up from slightly lower levels a week earlier. This drizzly weather also may have contributed to higher E. coli levels on both rivers from the first tests done two weeks ago.

The Merrimack River tested clean enough for swimming at all but two of the sites tested. The site at Goffs Falls near the railroad bridge and the site just upstream of where the Souhegan River comes into the Merrimack were 106 and 144 respectively, slightly higher than the 88 counts of E. coli bacteria colonies per 100 milliliters of water that is considered safe for public swimming areas in NH. All of the other sites between Manchester and Tyngesborough were below the 88 limit.

The Souhegan River, on the other hand, did not do as well. The fifteen miles of river upstream of the center of Milford tested clean. The fifteen miles downstream of the center of Milford did not fare so well. The site just upstream of the Oval, behind the Boys and Girls Club, tested at 188 colonies of bacteria and then the count jumped more than 100 to 292 just downstream of the Oval at the Swing Bridge. All of the readings downstream through Amherst and Merrimack also tested well over the 88 limit. The highest reading taken on the entire length of the river was 580 at the Amherst Canoeport on Rte 122 at the Amherst Country Club.

The site at the Boston Post Road Canoeport was not tested due to lack of volunteers, but it can be inferred that the water quality at that site was not healthy for swimming either. This is one of the popular summer swimming holes on the Souhegan. Anyone that would like to monitor one of the sites not presently covered may contact George May at 883-3409 or georgemay@adelphia.net. All of the sites showing NA, ‘not available,’ need volunteers. As one of the other volunteers put it, “It’s not hard.. .and it gives a good feeling to be helping out.” There are three sites on the Souhegan that need volunteers: one in Greenville, one in Merrimack, and one on Purgatory Brook in Mont Vernon, a major tributary of the Souhegan. There are also three sites that need volunteers on the Merrimack: one in Merrimack, one in Nashua, and one in Tyngesborough.

One of the surprises was the lower than expected count taken at the Downtown Wilton site. This is the second clean test taken at a site that has in the past always had very unhealthy water. This is also an area that is being looked at closely by the Souhegan Watershed Association to identify the DNA of bacteria that may be contaminating the river.

Drizzly weather and higher flows on these southern NH rivers didn’t deter people and animals from getting out and enjoying the rivers however. Monitors sighted fishermen, swimmers, dogs, birds, and turtles enjoying the brief respite between rainstorms early Tuesday morning. The Nashua Rowing Club has their dock back in the water just downstream of the Taylors Falls Bridge in Nashua and participants were out rowing early Tuesday morning. Unfortunately, one of the monitors also noted evidence of a horse who left behind a pile of manure that will be washed into the Nashua River and contribute to the E. coli counts after several rainstorms.

The rivers have also warmed up to pleasant temperatures for human activity. The Souhegan averaged 68 degrees; and the Merrimack was 70 degrees.

The Nashua River Watershed Association is monitoring the Nashua River once a month this year. The last test was on June 17. The NH sites monitored had generally higher E. coli counts than the May 27 tests. This was expected. The very high count noted in May at the canoe launch at Mines Falls Park was back to a more reasonable 99 in June.  All of the sites between Pepperell and the mouth of the river in Nashua were in the 125-130 range, marginally higher than acceptable for public swimming areas. Sites on the Nissitisit River, which flows from Lake Potanipo in Brookline into the Nashua River, tested very clean. Walker Brook in Mason, which flows into the Squannacook and then to the Nashua River, jumped from very low in the past to a higher, but still acceptable, 106. Generally all the Nashua River sites looked reasonably good this month.

The E. Coli samples are tested at the Milford, Merrimack, Nashua, and Pepperell Wastewater Treatment Facilities.

The next tests on the Merrimack and Souhegan will be done on July 11th. The results will be released as soon as the tests are completed. Testing takes 24 hours to grow the bacteria colonies.

The specific results for each test site follow:

SOUHEGAN RIVER SITES:

- SoR  333  River Road above Water Loom Pond, New Ipswich  49

- SoR  320  Highbridge, New Ipswich  8

- SoR  309  Above Greenville Mill Pond, Greenville  30

- SoR  296  Downtown Greenville upstream of WWTP  NA

- SoR  291  Green Bridge off Rte 31 below Greenville WWTP  74

- SoR  218  Below the Horseshoe, Wilton   32

- SoR  210  Downtown Wilton  50

- SoR  201  Pine Valley Mill, West Milford   28

- SoR  170  Behind Hayward Field, West Milford   31

- SoR  155  Souhegan Valley Boys and Girls Club, Milford  188

- SoR  146  Swing Bridge, Milford   292

- SoR  133  Riverside Cemetery, Milford   212

- SoR  130  Behind Lorden Plaza, Milford  264

- SoR  122  Amherst Country Club  580

- SoR  116  Amherst Conservation land Fairway Road, Amherst   440

- SoR  095  Boston Post Road Canoeport, Amherst  NA

- SoR  057  Indian Ledges, Merrimack  296

- SoR  034  Turkey Hill Bridge, Merrimack   380

- SoR  004  East Chamberlain Rd, Merrimack  NA

- Billy Ward Pond, Ashburnham  2

- Purgatory Brook, Lyndeborough/Mont Vernon  NA

MERRIMACK RIVER SITES:

- Mer  600  Above Amoskeag Dam, Manchester  24

- Mer  590  Arms Park, Manchester  68

- Mer  580  Upstream of Piscataquag River, Manchester    NA

- Mer  570  Goffs Falls, Litchfield  106

- Mer  560  Depot Street, Merrimack    38

- Mer  550  Upstream of Souhegan River, Merrimack   144

- Mer  540  Thorntons Ferry, Merrimack  NA

- Mer  530  Greeley Park, Nashua  38

- Mer  520  Taylors Falls Bridge, Nashua   55

- Mer  510  Sagamore Bridge, Hudson  16

- Mer  500  Pheasant Lane Mall, Nashua    NA

- Mer  490  Upstream of Tyngsboro Bridge  NA

NASHUA RIVER SITES

- Covered Bridge, Pepperell  126

- Canoe Launch, Pepperell  137

- Runnels Bridge, Hollis  124

- Mine Falls Park, Nashua  99

- Canal Street, Nashua  130

- Lancy Brook, Brookline  6

- Nissitisit River, Brookline  9

- Nissitisit River, West Hollis  32

- Walker Brook, Mason  106